# Conveyor station

# Solution to exercise 8: Sorting workpieces

## Learning objectives

Upon completing this exercise, you should

- be familiar with the function of inductive and opto sensors

- be able to realise control circuits using sensors

- be able to realise simple control systems using FluidSIM®

## Problem description

Conveying and sorting tasks are important functions in all production. This exercise consists of designing a conveyor and an accompanying control program with the following characteristics:

the workpieces (containers and lids in red or black) are to be conveyed from the start to the end of the conveyor.

Conveying should start when a workpiece is placed at the start of the conveyor and stop once the workpiece has left the conveyor at the other end.

Silver workpieces should be rejected onto the slide.

## Task

- What conveyor setup is needed to realise the specified functions? Create a schematic diagram of the setup showing the arrangement of each component and specify meaningful designations for the components.

- Create an allocation list that shows which slot on the multi-pin plug distributor the electrical components are plugged into.

- Set up the conveyor as per the schematic diagram and wire in accordance with the allocation list.

- Develop a control program using FluidSIM® to monitor the specified functions. Provide an ON/OFF switch. Test the program via simulation.

- Extend the FluidSIM® program to include the multi-pin plug distributor, set the labels as per the allocation list and test your program using the conveyor station.

## Aids

- Theory book

- FluidSIM®

- Conveyor station

Name:							Class:			Date:

- What conveyor setup is needed to realise the specified functions? Create a schematic diagram of the setup showing the arrangement of each component and specify meaningful designations for the components.

Note: Your schematic diagram can vary slightly; however, the position of the actuators and sensors must be clear and their designations must match the allocation list.

- Create an allocation list that shows which slot on the multi-pin plug distributor the electrical components are plugged into.

| Slot | Designation | Explanation |
| --- | --- | --- |
| 0 | IND | Inductive sensor |
| 1 | K1 | Solenoid/deflector relay output |
| 2 | OPT | Opto sensor |
| 3 | K2 | DC motor relay output |

Name:							Class:			Date:

- Develop a control program using FluidSIM® to monitor the specified functions. Provide an ON/OFF switch. Test the program via simulation.

Name:							Class:			Date:

- Set up the station as per the schematic diagram, install tubing between all pneumatic components and connect the electrical components to the multi-pin plug distributor. Carry out a functional test of the controller using the station.